Course Project – Module 4

College of Engineering and Information Sciences

Course Number: CEIS114

# Creating a Multiple Traffic Light Controller

# PARTS LIST

- PC running Arduino IDE

- ESP32 Board

- Two sets of Colored LEDs: Red, Yellow and Green

- Wires

- Breadboard

# Deliverables

- Complete the Course Project PowerPoint Deliverable

- Include a picture of your circuit

- Screenshot of Arduino IDE code from your computer

# PROCEDURE

- Connect the ESP32 Board and Check if the board is recognized by the Arduino IDE. If needed, please refer to Module 1 Project for connecting and configuring the ESP32 Board.

- Insert the three LEDs ( Red, Yellow, Green) in the breadboard as shown in Figure 1

- Connect all the wires as shown in Figure 1. Make sure to connect the Red, Yellow, and Green wires to the appropriate pins in the ESP32 Board.

- Open the Arduino IDE and load the code shown in Figure 2

Figure 1. Two Traffic Lights

Figure 2. Code for Two Traffic Lights (6 LEDs: Green, Yellow, Red)

// === Replace this text with your Name ====

// Module #4 project

// Define some labels

const int red_LED1  = 14;   // The red LED1 is wired to ESP32 board pin GPIO14

const int yellow_LED1  =12;   // The yellow LED1 is wired to ESP32 board pin GPIO12

const int green_LED1 = 13; // The green LED1 is wired to ESP32 board pin GPIO13

const int red_LED2  = 25;   // The red LED2 is wired to Mega board pin GPIO25

const int yellow_LED2  = 26;   // The yellow LED2 is wired to Mega board pin GPIO 26

const int green_LED2 = 27; // The green LED2 is wired to Mega board pin GPIO 27

// the setup function runs once when you press reset or power the board

void setup() {

pinMode(red_LED1, OUTPUT);  // initialize digital pin GPIO14 (Red LED1) as an output.

pinMode(yellow_LED1, OUTPUT);  // initialize digital pin GPIO12 (yellow LED1) as an output.

pinMode(green_LED1, OUTPUT);    // initialize digital pin GPIO13 (green LED1) as an output.

pinMode(red_LED2, OUTPUT);  // initialize digital pin GPIO25(Red LED2) as an output.

pinMode(yellow_LED2, OUTPUT); // initialize digital pin GPIO26 (yellow LED2) as an output.

pinMode(green_LED2, OUTPUT); // initialize digital pin GPIO27 (green LED2) as an output.

}

// the loop function runs over and over again forever

void loop() {

// The next three lines of code turn on the red LED1

digitalWrite(red_LED1, HIGH);      // This should turn on the RED LED1

digitalWrite(yellow_LED1 , LOW);        //  This should turn off the YELLOW LED1

digitalWrite(green_LED1, LOW);        //  This should turn off the GREEN LED1

delay(1000); //Extended time for Red light#1 before the Green of the other side turns ON

// The next three lines of code turn on the green LED2 for 2 seconds

digitalWrite(red_LED2, LOW);         // This should turn off the RED LED2

digitalWrite(yellow_LED2 , LOW);  //  This should turn off the YELLOW LED2

digitalWrite(green_LED2, HIGH);   //  This should turn on the GREEN LED2

delay(2000);                      // wait for 2 seconds

// The next three lines of code turn on the red LED1

digitalWrite(red_LED1, HIGH);          // This should turn on the RED LED1

digitalWrite(yellow_LED1 , LOW);         //  This should turn off the YELLOW LED1

digitalWrite(green_LED1, LOW);          //  This should turn off the GREEN LED1

// The next three lines of code turn on the yellow LED2

digitalWrite(red_LED2, LOW);         // This should turn off the RED LED2

digitalWrite(yellow_LED2 , HIGH);  //  This should turn on the YELLOW LED2

digitalWrite(green_LED2, LOW);   //  This should turn off the GREEN LED2

delay(2000);                     // wait for 2 seconds

// The next three lines of code turn on the red LED2

digitalWrite(red_LED2, HIGH);         // This should turn on the RED LED2

digitalWrite(yellow_LED2 , LOW);  //  This should turn off the YELLOW LED2

digitalWrite(green_LED2, LOW);   //  This should turn off the GREEN LED2

delay(1000); //Extended time for Red light#2 before the Green of the other side turns ON

// The next three lines of code turn on the green  LED1

digitalWrite(red_LED1, LOW);       // This should turn off the RED LED1

digitalWrite(yellow_LED1 , LOW);        //  This should turn off the YELLOW LED1

digitalWrite(green_LED1, HIGH);        //  This should turn on the GREEN LED1

delay(2000);                      // wait for 1 second

// The next three lines of code turn on the yellow  LED1

digitalWrite(red_LED1, LOW);      // This should turn off the RED LED1

digitalWrite(yellow_LED1 , HIGH);        //  This should turn on the YELLOW LED1

digitalWrite(green_LED1, LOW);        //  This should turn off the GREEN LED1

// The next three lines of code turn on the red LED2

digitalWrite(red_LED2, HIGH);         // This should turn on the RED LED2

digitalWrite(yellow_LED2 , LOW);  //  This should turn off the YELLOW LED2

digitalWrite(green_LED2, LOW);   //  This should turn off the GREEN LED2

delay(2000);                      // wait for 1 second

}